# Jaws and Braille SENSE Key define

This table defines function of commands from Braille Sense to Jaws.

# Movement Commands

| Jaws | Braille SESNE | Jaws | Braille SENSE |
| --- | --- | --- | --- |
| Top of file | Dots 1, 2, 3 + Space | Bottom of file | Dots 1, 2 Dots 4, 5, 6 + Space |
| Say Line | Dots 1, 4 + Space | Say Next Character | Dot 6 + Space |
| Say Next Line | Dot 4 + Space | Say Next Word | Dot 5 + Space |
| Say Prior Character | Dot 3 + Space | Say Prior Line | Dot 1 + Space |
| Say Prior Word | Dot 2 + Space | Say Word | Dots 2, 5 + Space |
| Say Character | Dots 3, 6 + Space | Select All (CTRL + A) | Dots 1, 8 |
| Braille Move Left | ADVANCE1 + ADVANCE2 | Braille Move Right | ADVANCE3 + ADVANCE4 |

# Application and Window Functions

| Jaws | Braille SESNE | Jaws | Braille SENSE |
| --- | --- | --- | --- |
| Toggle active cursor moves to Braille cursor | Dots 3, 6 | Cut selected text to clipboard (CTRL+X) | Dots 1, 3, 4, 6 |
| Minimize all applications | ADVANCE1 + ADVANCE4 | Copy selected text to clipboard (CTRL+C) | Dots 1, 4 |
| Read whole document (say all) | Dots 1, 2, 4, 5 | Paste from clipboard (CTRL+V) | Dots 1, 2, 3, 6 |
| Start menu | Dots 2, 3, 4 + Space | Menu bar (activates ALT key) | Dots 1, 3, 4 + Space ADAVNCE2 |
| Braille verbosity | Dots 1, 2, 3 | ESC | Dots 1, 5 + Space ADVANCE4 |
| Expand current Grade 2 word | Dots 2, 4, 5, 6 | Toggle Grade 2 Braille on/off | SCROLL_LEFT + SCROLL_RIGHT |
| Keyboard Help | Dots 1, 3 | Run JAWS Manager | Dots 2, 4 |
| JAWS Delete | Dots 1, 4, 5 + Space | JAWS Window | Dots 2, 4, 5 |

# QWERTY Keyboard Simulation

| Jaws | Braille SESNE | Jaws | Braille SENSE |
| --- | --- | --- | --- |
| Move to next field (TAB) | Dots 4, 5+ Space ADVANCE3 | Reverse tab in menus or dialog boxes (SHIFT+TAB) | Dots 1, 2 + Space Space + ADVANCE3 |
| Braille ALT +TAB | Dots 4, 5, 7 + Space ADVANCE2 + ADVANCE3 | Braille Enter | Dot 8 |
| Braille Toggle 8 Pixels Per Space | Dots 2, 3, 5, 6 | Move To Next Non-Link Text | Dots 4, 5, 6 |

# Toggle or Cycling Commands

| Jaws | Braille SESNE | Jaws | Braille SENSE |
| --- | --- | --- | --- |
| Toggles Cursor: PC cursor, or JAWS cursor | Dot 3 | Toggles whether active cursor follows Braille cursor | Dot 4 |
| Cycles Marking: bold, underline, italic, strikeout, all, off, highlight | Dot 2 | Toggles 6 or 8 dot Braille | ADVANCE1 + SCROLL_RIGHT |
| Cycles Mode: speech history, attribute mode, line mode, structured mode | Dot 1 |  |  |

# Front Panel Navigational Thumb Keys

| Jaws | Braille SESNE | Jaws | Braille SENSE |
| --- | --- | --- | --- |
| 32 characters BACK (Left) | SCROLL_LEFT | 32 characters FORWARD (Right) | SCROLL_RIGHT |
| Moves display UP one line (Prior) | ADVANCE4 + SCROLL_RIGHT | Moves display DOWN one line (Next) | ADVANCE1 + SCROLL_LEFT |

# Internet Explorer Keys

| Jaws | Braille SESNE | Jaws | Braille SENSE |
| --- | --- | --- | --- |
| Select a Frame | Dots 1, 2, 4 | Select a Link | Dots 2, 4, 7 |
| Go Forward to next web page | Dots 1, 3, 4, 5 Dots 6, 7 + Space | Go Back to prior web page | Dots 1, 2, 3, 4 Dots 3, 7 + Space |

# Outlook Keys

| Jaws | Braille SESNE | Jaws | Braille SENSE |
| --- | --- | --- | --- |
| Go to Attachments List | Dots 1, 2, 4 | Previous Message | Dot 1 + Space |
| Next Message | Dot 4 + Space | Delete Message | Dots 1, 4, 5 + Space |