- Connection Guide between JAWS & Braille Sense Display

- Exit JAWS if you are using it.

- Install the Braille Display Driver. (JawsPatch_for_BS.exe).

  - Please refer to the install guide for installing the Braille Sense display.

- Serial Interface.

  - Connect PC and the Braille Sense with serial cable that is included in package.

  - Execute ‘utility/terminal for screen reader’ program.

  - Select ‘serial port (s)’ to use serial interface.

  - Please make sure that the message is ‘terminal mode’.

  - Select serial port in your PC to connect the Braille Sense and JAWS. ( Jump to #6 )

- Bluetooth Serial Interface.

  - In order to use the Bluetooth serial port, please refer to the Bluetooth Connection Guide.

  - Select Bluetooth serial port in your PC to connect the Braille Sense and JAWS. ( Jump to #6 )

- USB interface.

  - If you have already installed the Braille display driver, USB connection driver will be installed in your PC.

  - Connect PC and the Braille Sense with USB cable that is included in package.

  - Execute ‘utility/terminal for screen reader’ program.

  - Select ‘usb port (u)’ to use USB interface.

  - Please make sure that the message is ‘terminal mode’.

  - Select USB port in your PC to connect the Braille Sense and JAWS.

- Execute JAWS.

  - Execute ‘File-> Braille'

  - Select Braille Sense and port that you want to use and press OK button.

  - That’s all that you can connect the Braille Sense to JAWS.