- Connection Guide between Supernova & Braille Sense Display

- Exit Supernova if you are using it.

- Install the Braille Display Driver (SuperNovaPatch_for_BS.exe).

  - Please refer to the install guide for installing the Braille Sense display.

- Serial Interface.

  - Connect PC and the Braille Sense with serial cable that is included in package.

  - Execute ‘utility/terminal for screen reader’ program.

  - Select ‘serial port (s)’ to use serial interface.

  - Please make sure that the message is ‘terminal mode’.

  - Detect Braille Display in your Supernova. ( Jump to #6 )

- Bluetooth Serial Interface.

  - In order use the Bluetooth Serial port, please refer to Bluetooth Serial connection Guide.

  - Detect Braille Display in your Supernova. ( Jump to #6 )

- USB interface.

  - If you have already installed the Braille display driver, USB connection driver will be installed in your PC.

  - Connect PC and the Braille Sense with USB cable that is included in package.

  - Execute ‘utility/terminal for screen reader’ program.

  - Select ‘usb port (u)’ to use USB interface.

  - Please make sure that the message is ‘terminal mode’.

  - Detect Braille Display in your Supernova.

- Execute Supernova.

  - Execute 'Supernova Control Panel->Configuration->SAM Setup'

  - Click 'Detect all devices now' button in 'Sam Configuration' dialog box.

  - That’s all that you can connect the Braille Sense to Supernova.